Numerical experiments on plasma focus neon soft x-ray scaling

S Lee1,2,3, S H Saw3, P Lee2 and R S Rawat2

1 Institute for Plasma Focus Studies, 32 Oakpark Drive, Chadstone, VIC3148, Australia

2 Nanyang Technological University, National Institute of Education, Singapore 637616

3INTI International University College, 71800 Nilai, Malaysia

e-mail: leesing@optusnet.com.au

Abstract

Numerical experiments are carried out systematically to determine the neon soft x-ray yield Ysxr for optimized neon plasma focus with storage energy E0 from 1 kJ to 1 MJ. The ratio c=b/a, of outer to inner radii; and the operating voltage V0 are kept constant. E0 is varied by changing the capacitance C0. Parametric variation at each E0 follows the order operating pressure P0, anode length z0 and ‘a’ until all realistic combinations of P0, z0 and ‘a’ are investigated. At each E0, the optimum combination of P0, z0 and ‘a’ is found that produces the biggest Ysxr. At low energies Ysxr~E01.4 whilst towards 1 MJ Ysxr~E00.7. The Ysxr scaling laws are found to be Ysxr~Ipeak3.2 (0.2 – 2.4 MA) and Ysxr~Ipinch3.6 (0.1-1.3 MA) throughout the range investigated. When numerical experimental points with other ‘c’ values and mixed parameters are included, there is evidence that the Ysxr vs Ipinch scaling is more robust and universal, remaining unchanged whilst the Ysxr vs Ipeak scaling changes slightly, with more scatter becoming evident.

Plasma focus machines operated in neon have been studied as intense sources of soft x-rays with potential applications.1-3  Whilst many recent experiments have concentrated efforts on low energy devices1-3   with a view of operating these as repetitively pulsed sources, other experiments have looked at x-ray pulses from larger plasma focus devices4,5  extending to the MJ regime. Numerical experiments simulating

x-ray pulses from plasma focus devices are also gaining more interest in the public domain. For example, the Institute of Plasma Focus Studies6 conducted a recent international Internet Workshop on Plasma Focus Numerical Experiments, 7  at which it was demonstrated that the Lee model code8 not only computes realistic focus pinch parameters, but also absolute values of soft x-ray yield Ysxr which are consistent with those measured experimentally. A comparison was made for the case of the NX2 machine,3  showing good agreement between computed and measured Ysxr as a function of P0.7,9 This gives confidence that the Lee model code gives realistic results in the computation of Ysxr. In this paper we report on a comprehensive range of numerical experiments with storage energies E0 in the range of 1 kJ to 1 MJ, in order to derive the scaling laws for Plasma focus  neon Ysxr, in terms of E0, peak discharge current Ipeak and focus pinch current Ipinch.

Numerical experiments to derive scaling laws on neutron yield Yn have already been reported. 10,11 These have shown that in terms of  storage energy E0, Yn~E02 at small E0 of kJ,  the scaling ‘slowing’ with increasing E0, becoming Yn~E0 in the higher energy ranges of MJ. In terms of Ipeak, a single power law covers the scaling, this being: Yn~Ipeak3.8; likewise another single power law for Ipinch, this being Yn~Ipinch4.5. These scaling laws apply from kJ to 25 MJ with corresponding Ipeak from 0.3-5.7 MA and Ipinch from 0.2-2.4 MA. It needs to be stressed that these scaling rules only apply to optimized operational points. It also needs to be pointed out that the distinction of Ipinch from Ipeak is of basic importance.12-14 . The scaling with Ipinch is the more fundamental and robust one;

since obviously there are situations (no pinching or poor pinching however optimized) where Ipeak may be large but Yn is zero or small; whereas the scaling with Ipinch is certainly more consistent with all situations. In  these works the primary importance of Ipinch for scaling plasma focus properties including neutron yield Yn, has been firmly established. 10-14

This primary importance of Ipinch has been borne in mind in our numerical experiments on neon plasma focus. In the context of neon Ysxr scaling, not much work appears to have been reported in the literature. D C Gates, in optimization studies had proposed15 that the total energy emitted as x-rays may scale as Yx~Ipeak4/(pinch radius)2. This scaling rule is not very useful for predictive purposes since for a given capacitor bank whilst Ipeak may be estimated, the focus pinch radius is difficult to quantify. Moreover if one considers a certain gas, say, neon, then for optimum operation one really needs to fix an axial speed, in which case the speed factor S=(Ipeak/a)/P00.5 (where ‘a’ is the anode radius and P0 is the operating pressure) is fixed.16 Moreover for optimum operation in neon, the pinch radius has a fixed relationship to ‘a’.17 This means that the Gates’ scaling rule reduces to Yx~P0Ipeak2. It is of greater interest to note that N V Filippov et al5 had compared the experimental data of two Filippov- type plasma focus operated at 0.9 MJ and 5 kJ respectively and on the basis of the experimental results of just these two machines had proposed a scaling for the K-shell lines of Neon Yx~Ipinch3.5-4. They further stated that such a scaling is in conformity to the resistive heating mechanism

of neon plasma. It is unlikely that Filippov’s Yx~Ipinch3.5-4 is compatible with Gates’ Yx~Ipeak4/(pinch radius)2. It is against this background of rather scanty experimental data that our numerical experiments are designed to comprehensively cover the range of E0 from 1 kJ to 1 MJ using the Lee model code which models Mather- type configurations.

The Lee model couples the electrical circuit with plasma focus dynamics, thermodynamics and radiation, enabling realistic simulation of all gross focus properties.

The basic model, described in 1984, 18 was successfully used to assist several

projects.19-21 An improved 5-phase model and code incorporating a small disturbance

speed22 and radiation coupling with dynamics assisted other research projects23-26

and was web-published in 200027 and 2005.28  Plasma self-absorption was included

in 200727 improving soft x-ray yield simulation. The code has been used extensively

in several machines including UNU/ICTP PFF, ,19,20,23,24,29,30  NX2,3,25,26  NX13 and adapted for the Filippov-type plasma focus DENA.31 A recent development is the inclusion of the neutron yield, Yn, using a beam–target mechanism, 10,11,13,32,33  incorporated in recent  versions8 of the code (later than RADPFV5.13), resulting in realistic Yn scaling with Ipinch. 10,11   The versatility and utility of the model is demonstrated in its clear distinction of Ipinch from Ipeak 12  and the recent uncovering of a plasma focus pinch current limitation effect.13,14  The description, theory, code and a broad range of results of this ‘Universal Plasma Focus Laboratory Facility’ is available for download from. 8

In the code, neon line radiation QL is calculated as follows

where for the temperatures of interest in our experiments we take Ysxr=QL

Hence the SXR energy generated within the plasma pinch depends on the properties:

number density ni, effective charge number Z, pinch radius rp, pinch length zf and

temperature T and pinch duration ; since in our code the QL is obtained by integrating over the pinch duration.

This generated energy is then reduced by the plasma self-absorption which depends primarily on density and temperature; the reduced quantity of energy is then emitted as the SXR yield. It was first pointed by Liu Mahe30 that a temperature around 300 eV is optimum for SXR production. Shan Bing’s subsequent work25 and our experience through numerical experiments suggest that around 2x106 K (below 200eV) seems to be better. Hence unlike the case of neutron scaling, for SXR scaling there is an optimum small range of temperatures (T window) to operate.

We use the Lee model code to carry out a series of numerical experiments over the energy range 1 kJ to 1 MJ. For neon operation, the Lee model code had previously been designed to compute the line radiation yield. For this work we want to distinguish that part of the line yield that is soft x-rays. Reviewing previous experimental and numerical work by Liu30  and more detailed numerical work by Shan Bing25 we are able to fix a temperature range for neon at which the radiation is predominantly soft x-ray

coming from He-like and H-like neon ions. Shan Bing in particular carried out a line-by-line computation using a corona method and computed the relative intensities of each of 4 neon soft x-ray lines (He- and H-like) as functions of temperature. From this review we set the following temperature range: 2.3-5.1x106 K as that relevant to the production of neon soft x-rays. In any shot, for the duration of the focus pinch, whenever the focus pinch temperature is within this range, the line radiation is counted as neon soft x-rays. Whenever the pinch temperature is outside this range, the line radiation is not included as neon soft x-rays.

The following parameters are kept constant. These are the ratio b=c/a (kept at 1.5) and the operating voltage V0 (kept at 20 kV), static inductance L0 (kept at 30 nH) and the ratio of stray resistance to surge impedance RESF (kept at 0.1) . The model parameters7,8,10-14 fm, fc, fmr, fcr are also kept at fixed values 0.06, 0.7, 0.16 and 0.7.

The storage energy E0 is changed by changing the capacitance C0. Parameters that are varied are operating pressure P0, anode length z0 and anode radius ‘a’. Parametric variation at each E0 follows the order operating pressure P0, anode length z0 and ‘a’ until all realistic combinations of P0, z0 and ‘a’ are investigated. At each E0, the optimum combination of P0, z0 and ‘a’ is found that produces the biggest Ysxr. In other words at each E0, a P0 is fixed, a z0 is chosen and ‘a’ is varied until the largest Ysxr is found. Then keeping the same values of E0 and P0, another z0 is chosen and ‘a’ is varied until the

largest Ysxr is found. This procedure is repeated until for that E0 and P0, the optimum combination of z0 and ‘a’ is found. Then keeping the same value of E0, another P0 is selected. The procedure for parametric variation of z0 and ‘a’ as described above is then carried out for this E0 and new P0 until the optimum combination of z0 and ‘a’ is found. This procedure is repeated until for a fixed value of E0, the optimum combination of P0, z0 and ‘a’ is found.

The procedure is then repeated with a new value of E0. In this manner after systematically carrying out some 2000 shots, the optimized shots for various energies are tabulated in Table 1.

Table 1 Optimised configuration found for each E0. Optimisation carried out with RESF=0.1, c=1.5, L0=30nH and V0=20 kV and model parameters fm, fc, fmr, fcr are fixed at 0.06, 0.7, 0.16 and 0.7 respectively.  va, vs and vp are the peak axial, radial shock and radial piston speeds respectively

| E0 (kJ) | C0 (F) | a (cm) | z0 (cm) | P0 (Torr) | Ipeak (kA) | Ipinch (kA) | va (cm/s) | vs (cm/s) | vp (cm/s) | Ysxr (J) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 5.0 | 1.1816 | 1.5 | 4.0 | 224 | 143 | 6.6 | 23.3 | 15.1 | 7.51 |
| 2.0 | 10.0 | 1.524 | 2.1 | 4.0 | 300 | 186 | 6.8 | 23.6 | 15.2 | 20.4 |
| 6.0 | 30.0 | 2.285 | 5.2 | 4.2 | 512.0 | 294 | 8.1 | 24.5 | 15.6 | 97.5 |
| 10.0 | 50.0 | 2.785 | 7.5 | 4.0 | 642 | 356 | 8.7 | 24.6 | 15.7 | 194.8 |
| 20.0 | 100 | 3.499 | 13.0 | 4.0 | 861 | 456 | 9.6 | 24.6 | 16.0 | 469.1 |
| 40.0 | 200 | 4.551 | 20.0 | 3.5 | 1109 | 565 | 10.3 | 24.7 | 16.2 | 1030 |
| 100 | 500 | 6.213 | 42.0 | 3.0 | 1477 | 727 | 11.2 | 24.8 | 16.4 | 2692 |
| 200 | 1000 | 7.415 | 63.0 | 3.0 | 1778 | 876 | 11.4 | 24.8 | 16.5 | 5292 |
| 400 | 2000 | 8.700 | 98.0 | 3.0 | 2079 | 1036 | 11.4 | 24.9 | 16.5 | 9379 |
| 500 | 2500 | 9.100 | 105 | 2.9 | 2157 | 1086 | 11.5 | 25.1 | 16.7 | 11095 |
| 1000 | 5000 | 10.20 | 160 | 3.0 | 2428 | 1261 | 11.4 | 25.2 | 16.7 | 17674 |

From the data of Table 1, we plot Ysxr against E0 as shown in Fig 1

Fig 1. Ysxr vs E0. The parameters kept constants are: RESF=0.1, c=1.5, L0=30nH and V0=20 kV and model parameters fm, fc, fmr, fcr at 0.06, 0.7, 0.16 and 0.7 respectively

Fig 1 shows that Ysxr scales as E01.44 at low energies in the 1 to 2 kJ region. The scaling ‘drops’ as E0 is increased, and Ysxr scales as E00.67 at high energies towards 1MJ.

We then plot Ysxr against Ipeak and Ipinch and obtain Fig 2

Fig 2. Ysxr vs Ipinch, Ipeak. The parameters kept constants are: RESF=0.1, c=1.5, L0=30nH and V0=20 kV and model parameters fm, fc, fmr, fcr at 0.06, 0.7, 0.16 and 0.7 respectively

Fig 2 shows that Ysxr~Ipinch3.6 and Ysxr~Ipeak3.2. The Ipinch scaling has less scatter than the Ipeak scaling.

We next test the scaling when the fixed parameters RESF, c=1.5, L0 and V0 and model parameters fm, fc, fmr, fcr are varied. We add in the results of some numerical experiments using the parameters of several existing plasma focus devices including the

UNU/ICTP PFF (RESF=0.2, c=3.4, L0=110 nH and V0=14 kV with fitted model

parameters fm=0.05, fc=0.7, fmr=0.2, fcr=0.8),  the NX2 (RESF=0.1, c=2.2, L0=20 nH and V0=11 kV with fitted fm=0.06, fc=0.7, fmr=0.16, fcr=0.7) and PF1000 (RESF=0.1, c=1.39, L0=33 nH and V0=27 kV with fitted model parameters fm=0.1, fc=0.7, fmr=0.15, fcr=0.7).  These new data points contain wide ranges of ‘c’, V0, L0 and model parameters. The resulting Ysxr vs Ipinch log-log curve remains a straight line, with the scaling index 3.6 unchanged and  with no more scatter than in Fig 2.  However the resulting Ysxr vs Ipeak log-log curve now exhibits even larger scatter than in Fig 2 and the scaling index has changed to 3.1.

Another way of looking at the comparison of the Ipinch scaling and the Ipeak scaling is to consider some unoptimised cases e.g. at very high or very low pressures. In these cases, Ysxr is zero and Ipinch is zero but there is a value for Ipeak . This is an argument  that the Ipinch scaling is more robust. However it must be noted that  both scaling are applicable only to optimized points. Nevertheless noting that the Ysxr~Ipinch scaling has less scatter than the Ysxr~Ipeak scaling particularly when mixed-parameters cases are included, the conclusion is that the Ipinch scaling is the more universal and robust one.

In summary, numerical experiments with neon plasma focus over storage energy range of 1 kJ to 1 MJ show that within the stated constraints of these experiments, scaling with E0 is Ysxr~E01.4 in the low energy range toward 1 kJ and ‘decreases’ to Ysxr~E00.7 in

the high energy range investigated towards 1 MJ. A single power law applies for the Ipeak scaling: Ysxr~Ipeak3.2, in the range of 0.2-2.4 MA;  likewise for Ipinch scaling: Ysxr~Ipinch3.6,

in the range 0.1-1.3 MA. The observations of the numerical experiments, bolstered by fundamental considerations is that the Ipinch scaling is the more universal and robust one. It may also be worthy of note that our comprehensively surveyed numerical experiments for Mather configurations in the range of energies 1 kJ to 1 MJ produces an Ipinch scaling

rule not compatible with Gates’ rule. 15  However it is remarkable that our Ipinch scaling index of 3.6, obtained through a set of comprehensive numerical experiments over a range of 1 kJ to 1 MJ, on Mather-type devices is within the range of 3.5-4 postulated on the basis of not so comprehensive experimental data, (basically just two machines one at 5 kJ and the other at 0.9 MJ), by Filippov5, for Filippov configurations in the range of energies 5 kJ to 1 MJ.

References

1 Y Kato and S H Be. 1986 Appl. Phys. Lett. 48 686

2E P Bogolyubov, V D Bochkov, V A Veretennikov, L T Vekhoreva, V A Gribkov, A V Dubrovskii, Yu P Ivanov, A I Isakov, O N Krokhin, P Lee, S Lee, V Ya Nikulin, A Serban, P V Silin, X Feng and G X Zhang1998 Phys. Scr. 57 488-494

3S Lee, P Lee, G Zhang, X Feng, V A Gribkov, M Liu, A Serban and T Wong. 1998 IEEE Trans. Plasma Sci. 26 1119

4N V Filippov, T I Filippova, M A Karakin, V I Krauz, V P Tykshaev, V P Vinogradov, Y P Bakulin, V V Timofeev, V F Zinchenko, J R Brzosko, J S Brzosko 1996 IEEE Trans Plasma Sc 24  1215 – 1223

5N. V. Filippov, T. I. Filippova, I. V. Khutoretskaia, V. V. Mialton and V. P. Vinogradov  1996  Physics Letters A  211, 168-171

6Institute for Plasma Focus Studies http://www.plasmafocus.net

7Internet Workshop on Plasma Focus Numerical Experiments (IPFS-IBC1) 14 April-19 May 2008    http://www.plasmafocus.net/IPFS/Papers/IWPCAkeynote2ResultsofInternet-basedWorkshop.doc

8S Lee  Radiative Dense Plasma Focus Computation Package: RADPF http://www.intimal.edu.my/school/fas/UFLF/File1RADPF.htm

http://www.plasmafocus.net/IPFS/modelpackage/File1RADPF.htm

9S Lee, R S Rawat, P Lee, S H Saw. Soft x-ray yield from NX2 plasma focus- correlation with plasma pinch parameters (to be published)

10S Lee and S H Saw 2008 Neutron scaling laws from numerical experiments J Fusion Energy 27 292-295

11S Lee 2008 Plasma Phys. Control. Fusion 50 105005

12S Lee, S H Saw, P C K Lee, R S Rawat and H Schmidt 2008 Appl. Phys. Lett. 92 111501

13S Lee and S H Saw 2008 Appl. Phys. Lett. 92 021503

14S Lee, P Lee, S H Saw and R S Rawat 2008 Plasma Phys. Control. Fusion 50 065012

15D C Gates 1978 Proceedings of the IInd Int Conference on Energy Storage, Compression and Switching, Venice, 2 329 (Plenum Press, New York, 1983)

16S Lee and A Serban A 1996 IEEE Trans. Plasma Sci. 24 1101

17S Lee S 1998 Scaling of the plasma focus. Int. Workshop on Plasma Focus Research (PF98) (Kudowa, Poland, July 1998) http://eprints.ictp.it/109/ (also in First Conf. on

Plasma Physics Applications, Int. Cooperation Bilateral Seminars vol 34 2004 Forschungszentrum Julich Gmbh (Julich, Germany) pp 27–33)

18S Lee 1984 Radiations in Plasmas ed B McNamara (Singapore: World Scientific) pp 978–87

19S Lee S et al 1988 Am. J. Phys. 56 62

20T Y Tou, S Lee and K H Kwek 1989 IEEE Trans. Plasma Sci. 17 311

21S Lee 1991 IEEE Trans. Plasma Sci. 19 912

22D E Potter 1971 Phys. Fluids 14 1911

23S Lee and A Serban 1998 J Plasma Phys 60:3-15

24M H Liu, X P Feng, S V Springham and S Lee 1998 IEEE Trans. Plasma Sci. 26 135–140

25S Bing 2000 Plasma dynamics and x-ray emission of the plasma focus PhD Thesis NIE ICTP Open Access Archive: http://eprints.ictp.it/99/

26D Wong, P Lee, T Zhang, A Patran, T L Tan, R S Rawat and S Lee 2007 Plasma Sources Sci Technol. 16 116-123

27S Lee 2000–2007 http://ckplee.myplace.nie.edu.sg/plasmaphysics/

28 S Lee 2005 ICTP Open Access Archive: http://eprints.ictp.it/85/

29 S Lee 1998 Twelve Years of UNU/ICTP PFF—A Review IC 98 (231) Abdus Salam ICTP, Miramare, Trieste; ICTP OAA: http://eprints.ictp.it/31/

30Liu Mahe (1996) Soft X-rays from Compact Plasma Focus. PhD thesis, Nanyang Technological University Singapore. ICTP Open Access Archive: http://eprints.ictp.it/327/

31V Siahpoush, M A Tafreshi, S Sobhanian and S Khorram 2005 Plasma Phys. Control. Fusion 47 1065

32 V A Gribkov et al 2007 J. Phys. D: Appl. Phys. 40 3592

33Springham S V, Lee S and Rafique M S. 2000 Plasma Phys.

Control. Fusion 42 1023