Simulations of a Ratiometric Switching Radiometer for Radio Astronomy

Marcus Leech, VE3MDL

# Introduction

Switched radiometers, such as those described by Dicke can be used to compensate for changes in the operating characteristics of a microwave radiometer. Such operating characteristics as gain and total system noise temperature can be affected by ambient temperature variations, and power-supply effects. It is typically the case that such variations, if uncompensated in some way, can exceed the magnitude of the microwave signals under measurement.

The Dicke radiometer uses a calibration source of known intensity in front of the first low-noise-amplifier (LNA) in the gain chain of a radiometer instrument. The input of the first LNA is alternately switched between the calibration source and the source to be measured. If the smoothed average of both of these virtual channels are obtained, then simple algebraic manipulations can be used to eliminate gain changes in computing measurements. A Dicke radiometer computes an approximation:

Output = ((Tsys + Tcal) * Gsys) - ((Tsys + Tsky) * Gsys)

Where: Tsys = system noise temperature, which varies somewhat with temperature

Tcal = calibration noise temperature

Tsky = sky noise temperature

Gsys = system gain, which varies significantly over temperature

In any given installation, Tcal is known within reasonable bounds, and Tsys varies only slightly over temperature, which means that the average Tsys value can be used as an estimate during measurement. If the ambient temperature of the first LNA(s) can be measured frequently, then better estimates of Tsys can be produced using laboratory-measured data for the front-end LNA(s). The instantaneous Gsys can be computed using the data from the virtual channels, and the known value for Tcal and Tsys estimate, yielding a good estimate for Tsky.  Many such estimates for Tsky can be averaged together to produce a high-quality estimate.

The Dicke radiometer is not without its limitations. Since a Dicke-switched radiometer must necessarily spend some significant amount of its time (50% is typical) “looking” at the calibration source, then a Dicke radiometer is half as sensitive as a comparable uncompensated radiometer. A further difficulty is that switching components for microwave frequencies that are both extremely-low-loss, and long-lived, are very difficult to procure.  Further, measurement error increases as the calibration signals differ in magnitude from the signal to be measured. In many radio-astronomy applications, the calibration source is adjusted so that its magnitude is roughly equivalent to the astronomical source under study.  This often requires complex and expensive equipment.

# The ratiometric switched radiometer

A new approach to the gain-drift problem has been studied, with simulation results that appear to be promising. The so-called ratiometric approach was originally suggested by Paul Shuch, of the SETI League. In this approach, a calibration source is used, but it is connected to the rest of the system in a different way. Importantly, there is no switching apparatus in front of the first LNA.

The stable calibration source is coupled into the front-end through the use of a directional coupler, with between 20dB and 30dB of isolation to the calibration source. The calibration source is simply switched on and off at regular intervals, in an identical fashion to the Dicke switching arrangement.

Such a ratiometric approach computes an approximation:

Output = ((Tsys + Tsky)*Gsys) / ((Tsys + Tsky + Tcal)*Gsys)

Importantly, Tcal is never isolated from Tsky, which means that this scheme produces an approximation whose quality depends at least partially on the relative magnitudes of Tsky and Tcal.

Just as in the Dicke system, Tcal should be known to within reasonable bounds, and Tsys can be estimated based on laboratory measurements, and optionally a real-time measurement of ambient temperature to improve the real-time Tsys estimate.  It can be seen from the expression above that this system is ratiometric. The Gsys term disappears in computing the ratio :

(Tsys+Tsky)/ (Tsys+Tsky+Tcal)

By selecting Tcal appropriately, and applying slope and intercept adjustments to the resulting data, measurement errors can be minimized over wide ambient temperature ranges.

# A simulation

A computer simulation was created, based on models of gain and noise-figure variability over temperature for a typical pHEMT amplifier, over the -40C to 45C temperature range. This simple model assumes linear characteristics for both gain and noise-figure over temperature.

The simulation varied Tsky between 6K and 18K, based on reasonable values to be expected for an amateur radio astronomy observatory operating in the 21cm region. The simulation also varied the ambient temperature between -40C and 45C, the extremes of ambient temperature ranges to be expected for an amateur radio observatory with no, or minimal, temperature control for the front-end LNA electronics. After some experimentation, a value for Tcal of 300K was chosen for the simulation runs. For simplicity, the simulation models an LNA with unity gain at 20C, and varying by 30% over the entire -40C to 45C temperature range, using an inverse-linear relationship between LNA gain and LNA ambient temperature.

An optimal slope and intercept calculation was performed to improve the linearity and accuracy of the measurement simulation, which produced the following graphical results:

The nearly-linear line shown is the average derived measurement of Tsky, based on the measurements of the simulated device under various temperature conditions from -40C to 45C.  The non-linear curve shown gives the measurement error, as a percentage, over the same simulated temperature range. The worst-case average error is at Tsky=6K, near the bottom of the range, which would produce peak errors (twice the average) of roughly 22% over the -40C to 45C temperature range.

# Conclusions

The simulation results provide a promising direction for future hardware development. For amateurs operating on tight budget constraints, this approach may be entirely appropriate, since SPDT microwave relays that have long lifetimes are costly.  This approach allows the power to the Tcal source to be switched with a Bipolar or Mosfet transistor, driven by a simple TTL signal. Directional couplers (known as “taps” in the cable industry) can be purchased from the cable-TV-distribution market quite cheaply.

Since signal loss in front of the first LNA contributes directly to the total  system noise temperature, it is desirable to minimize this loss to the greatest extent possible. An ideal switching mechanism would have no loss at all.

It is important that Tcal be stable over the expected ambient temperature range of the system. A simple technique such as using a small PTC-based oven can be used.